A6

Stereo Amplifier

| Product Description: The A Series 2U professional amplifiers have been engineered for reliability where full power is required for extended times in fixed installations or portable applications. The A Series delivers extraordinary performance at unmatched value. Key Features and Benefits: - Housed in a rugged, all-steel 2U chassis - 2 x convenient front panel carry handles - Rear panel selection switch for Stereo, Parallel and Bridged modes, including Low-cut filter, Ground lift and Limiter switches - Operates within 8Ω and 4Ω loads - Innovative heatsink with front to back air flow for efficient cooling - Variable fan speed is automatically adjusted by temperature - Electronically balanced XLR inputs - Touch proof binding post and Speakon outputs - Precision indented level controls, power switch, - Protect, Peak and Signal LEDs indicators for each channel - Amplifier protection includes short circuit, excessive temperature, and DC on outputs Front Panel: Rear Panel: Specifications:. Rated Power (Stereo 8Ω); 2 x 330W. Rated Power (Stereo 4Ω); 2 x 500W. Rated Power (Bridge 16Ω); 660W. Rated Power (Bridge 8Ω); 1000W. Rated Power (Parallel mono 8Ω); 350W. Rated Power (Parallel mono 4Ω); 600W. Input Sensitivity; 1V. S/N Ratio (A Weight); >108dB. Frequency Response (+0/-0.25dB); 20Hz-20kHz. Damping Factor; >800 @ 8ohm/100Hz. THD (10% Rated Power); <0.05%. Input Connector; 2 x XLR, 2 x 6.5mm TRS. Input Impedance; 20kΩ Balanced, 10kΩ Unbalanced. Output Connectors; 2 x NL4 + 4 x Binding post. Cooling; 4 x fans, airflow from front to back. Power Supply; 220V-230V 50/60Hz, 500VA. Dimensions (W x D x H); 483mm (19”) x 452mm (17.8”) x 88mm (3.5”). Packaging Dimensions (1 piece/pack); 550mm (21.7”) x 540mm (21.3”) x 175mm (6.9”). Net Weight (1 piece/pack); 15kg (33.1lb). Gross Weight (1 piece/pack); 17kg (37.5lb). | Mouseover image to zoom Front View Back View Planform. |
| --- | --- |